LIQUID TEST STAND PROCEDURES AND VALVE SEQUENCES

Default state prior to filling IPA tank:

- FV-110: Closed

- FCV-107A: Closed

- FCV-109: Closed

- FV-101: Open (probably either)

- FCV-104A: Closed

- FCV-104B: Closed

- FCV-102: Closed

- FCV-107B: Closed

- FCV-105A: probably either

- FCV-105B: probably either

Fill IPA tank:

- Open FV-101.

- Ensure FV-110 is closed.

- Ensure FCV-107A, FCV-109, and FCV-107B are closed.

- Ensure FCV-104A is closed.

- Fill IPA tank to desired level by pouring in IPA from the fitting on top of the tank. Once IPA tank is filled, close FV-101.

- Set the regulator on the N2 bottle to desired pressure to pressurize the IPA tank.

- Open FCV-109 and FCV-107A to pressurize the IPA tank.

Pressurize nitrous line:

- Ensure nitrous line has already been purged.

- Ensure FCV-107B and FCV-104B are closed.

- Open FCV-102.

Fire:

- Set needle valves to initial value.

- Enter startup sequence. The following things happen very quickly/overlapped in time:

  - Open FCV-104A and FCV-104B at the same time.

  - Fire ignitor.

  - Ramp up needle valve openings if needed (for gradual start).

Immediately post-fire:

- Close FCV-102 and FCV-104A.

Post-fire purge:

- Ensure FCV-102 is closed.

- Open FCV-104A, FCV-104B, FCV-107A, and FCV-107B simultaneously, purging lines with the remaining N2 pressure in the IPA tank.

- If additional purging is needed using gas from the N2 tank:

  - Open FCV-109, and once finished, close FCV-109.

Post-purge return to default state:

- Ensure FCV-109 is closed.

- Close FCV-107A, FCV-107B.

- Close FCV-104A, 104B.

- Optionally close FCV-105A, FCV-105B.

Approach the test stand:

- Visually verify safe state.

- Open FV-110 if needed to drain out any remaining fuel/residue. Close FV-110 once finished.

- Ensure test stand is ready for next fire.